- SERVICE MANUAL

- Repair for Cellular Telephone

- XENIUM DUAL BAND

LEVEL 1

- Service Manual

- Last updates:

| DATE | MODIFICATION | PAGE |
| --- | --- | --- |
| 19/06/2000 | CREATION |  |
| Revision 2 : 26/07/2000 | - Modification of components list | Page 23 |

- CONTENTS

1.0	PURPOSE	4

2.0	SCOPE	4

3.0	REFERENCE	4

4.0	Glossary/Acronym List	4

5.0	Test Equipment and Tools	4

6.0	Test and inspection Plan	5

6.1	User Interface Test	5

6.2	RF Test	5

7.0	Before Starting	6

7.1	Description of the transceiver	6

7.2	Description of the display	7

7.3	Using The Carousel	8

7.4	Inserting the MICRO-SIM card	9

7.5	Inserting on the battery	9

7.6	Removing the battery	10

7.7	Charging the battery	10

8.0	TEST PROCEDURES	11

8.1	Initial Functional check for TCD989/J XENIUM	11

8.2	RF TEST	15

8.3	Charging IGN (Ignition) – Battery	18

9.0	assembly / dismantlement procedures	19

9.1	Dismantlement	19

9.2	Assembly	19

10.0	Defaults Settings	20

10.1	Reset customer parameters.	20

10.2	Use of the GSM string *#RSAV*#  or *#7728*#.	20

11.0	SOLUTIONS IN CASE OF PROBLEMS DURING THE TESTS	21

11.1	The phone does not switch on.	21

11.2	Charge does not start or no detection of the charger.	21

11.3	The display shows “No SIM card. Please insert your SIM card.” or  “SIM FAILURE”	21

11.4	Display problems	22

11.5	Buzzer problems	22

11.6	No sound in Loudspeaker	22

11.7	Communication problems	22

11.8	Defective antenna	22

11.9	Keyboard problems	22

11.10	Problems to send SMS messages	22

12.0	recommended PART LIST- tcd 989 XENIUM DB	23

12.1	Common parts – out of warranty	23

# PURPOSE

This document establishes the functional test and inspection procedures for the first level service repair of the XENIUM DB transceiver

# SCOPE

The test plan is applicable to all levels of service repair of the XENIUM DB transceiver

# REFERENCE

# Glossary/Acronym List

| Window or Bezzel | Protective plastic over the LCD display |
| --- | --- |
| SW | Software |
| PN | Hardware Configuration of the Mobile |
| CN | Matrix for Types of SW used on the different hardware |
| HW | Hardware |
| ASC | Authorized Service Center |
| NSC | National Service Center |
| Test SIM Card | Used for functionality of PHILIPS Mobiles |
| Test SIM Card “SP” | SIM Card that is used to stimulate the user interface and allow radio tests |

# Test Equipment and Tools

- Equipment / Tools

Production Test SIM Card	- Part No. : 4311 255 00781

Test SIM Card “SP”		- Part No. : 4311 255 00782

RF Cable 			- Part No. : 941-555-1 (AMP).

Digital Multimeter		- Recommended Model : Fluke

Specification with current reading in mA.

Digital Radiocommunication Tester.

# Test and inspection Plan

The test plan is derived from the Product Test Reference for XENIUM DB.

## User Interface Test

Use the Test SIM card “SP” / Production to test the transceiver as follows :

- On/off Button

- LCD Backlight

- Keyboard Test

- Buzzer Test

- Audio Test

- Antenna Test (levels 5 &10)

- LCD

- LED Test (On/Off)

- IMEI

- Tester Status/Eeprom Status

With a fast charger connected with the PRODUCT’s bottom connector, check the full scrolling from one mode to the next when charging IGN (Ignition)-Battery.

## RF Test

The radio test must be performed with a Digital Radio Test Set connected to the mobile RF connector with the specific RF cable

# Before Starting

## Description of the transceiver

## Description of the display

Indicates the Network on which

your phone is registered.

Clock : Indicates an hour.

Reception quality : The more bars shown

the better the reception.

Indicates the date

Battery level : The bars indicate the battery level

(4 Bars means full, no bars means recharging is

needed.)

## Using The Carousel

The carousel is a circular loop of icons displayed on the screen. These icons provide access to the different menus and sub menus used to operate your phone.

## Inserting the MICRO-SIM card

### The mobile supports only the mini “plug-in” SIM card.

Push the metal retaining clip to the right and lift the cardholder. Slide in the SIM card between the retaining        clip and the plastic tongue with the cut corner of the card at the top left. Close the cardholder and push the retaining clip to the left.

## Inserting on the battery

### Place the battery on the back of the phone (connectors downward, the top near the arrow inside the case).

### Then push the battery into place in the direction of the antenna.

## Removing the battery

### Press the locking button located alongside the antenna while pushing the battery in the direction of the arrow.

### Remove battery.

## Charging the battery

### Plug the battery onto the transceiver

### Plug the charger into the connector at the base of the transceiver.

### Plug the transformer unit into the main AC power sockets.

### The battery charge symbol indicates the state of the charge process :

- Bars moving	-	means the battery is being charged

- Steady		-	means the battery is fully charged

If the battery is totally discharged, the battery icon will show and start scrolling 2 to 3 minutes only after connecting to the charger.

# TEST PROCEDURES

## Initial Functional check for TCD989/J XENIUM

### Insert the Test Production Card into the SIM Reader at the back of the cellular phone and clip a charged battery on the phone.

### Press the «ON» button for 2 seconds at least and the LCD will show a message which contains information of  FA and 12NC. (see

### Follow the instructions as mentioned below :

| Step | Procedure |  | Observation |
| --- | --- | --- | --- |
| 1 | Press Key 1 Press Key 1 again |  | Continue Buzzer signal Left corner displays 1 00 |
| 2 | Press key 2 (Audio loop local effect) Press key 2 again |  | "LocalEffect" " XX XX “ Left corner displays 2 01 |
| 3 | Press key 3 Audio loop test (Speak to Mic and listen echo from Speaker) Press key 3 again. |  | "AUDIO xx xx xx ” "EEP x xx xx xx ” Left corner displays 3 02 |
| 4 | Press key 4 (LEDs Test) Check for the Backlight function in the same time. Press key 4 again |  | Red and green LED blinking Left corner displays 4 03 |
| 5 | Press Key 5 (Checkerboard test) Press Key 5 again |  | Checkerboard 1 pixel on Left corner displays 5 04 |
| 6 | Press Key 6 (Inverted Checkerboard) Press Key 6 again |  | Checkerboard 2 pixel on Left corner displays 6 05 |
| 7 | Press Key 7 Press key 7 again |  | All pixels and hard icons on Left corner displays 7 06 |
| 8 | Press key 8 (Eeprom Status) Press Key 8 again |  | "EEPROM STAT” (Must be Good) H-XXXX-XXXX (Eeprom Status) L-XXXXXX-XX SimLk XXXXX (Sim lock Status) Left corner display 8 07 |
| 9 | Press Key 9 Product information Compare information with label printed on back case Press key 9 again |  | “PROD INFO” “XXXXXXXXX” (Product 12NC) “XXXXXXXX” (PN Number) VY made in Le Mans SA made in Singapore EO made in Shenzhen Left corner displays 9 08 |
| 10 | Press key 0 Press key 0 again |  | “ADC MEASURES” “XXXX XXXX” “XXXX XXXX” Left corner displays 0 09 |
| 11 | Press * (IMEI Test) Compare IMEI with label printed on back case Press * again |  | ###### "IMEI TEST" " XXXXXX/ 50 / XXXXXXX" for (TCD989/J) 06 made in Singapore 50 made in Le-Mans 69 made in China Left corner displays * 12 |
| 12 | Press # (FA Status) Press # again |  | "FA/12NC” FA GOOD (Must be good) X XXXXXXXXXXX (12NC) Left corner displays # 13 |
| 13 | Press C Press C again |  | Key without Test Left corner displays C 14 |
| 14 | Go to the UP with the Scanswitch (Melody Test) Go to the UP again |  | User Melody Left corner displays 0A |
| 15 | Go to the DOWN with the Scanswitch (Memory Test) Go to the DOWN again |  | “MEMORY TEST” “XXXXXXXX” “XXXXXXXX” “RAM OK” Left corner displays 0B |
| 16 | Press the Scanswitch Press the Scanswitch |  | “PAGE” “SELECTION” “XX” Left corner display OK 0E |
| 17 | Press Green button Press Green button again |  | “ MANUAL TEST” “ GOOD “ Left corner displays 0F |
| 18 | Press Red button Press Red button again |  | “ MANUAL TEST” “ BAD “ Left corner displays 10 |

    - If any of the step failed functional, refer to Chapter 10.

    - Perform visual check on battery connectors, car kit connectors and casing.

If corrosion or deform send to NSC for repair.

    - If the product is good, it is considered as a NFF product.

All the NFF products must be directly returned to the customer.

## RF TEST

### The Test SIM Card “SP” must be inserted into the phone before starting the tests.

### Set the equipment as shown on the picture in chapter 6.2.

### Set in the offset field of the radio tester a – 0.3 dBm lose for GSM Test

### The following operations must be done:

-	Synchronization/Registration

-	Call set up from the mobile

-	Voice loopback ( to check the sound quality)

-	Call release

-	Call set up from tester

- Call release from tester

### The following parameters must be checked in TCH loop mode :

Emission parameters :

-	Power level

-	RMS phase error

-	Peak phase error

-	Frequency error

-	Power ramping

- Reception parameters :

-	Rx level

-	Rx quality

-	BER (Byte Error Rate)

-	FER (Frequency Error Rate)

Generally the test sequences built inside the testers will be used to check the mobile. You must assess that the test sequences limits comply with the standard specifications.

### GSM Specification (900 Mhz)

| Test Parameters | Channel | Level | Standard Specification |
| --- | --- | --- | --- |
| EMISSION |  |  |  |
| Phase Error RMS | 1, 62, 124 | 5, 10, 15 | 0 to 5 degrees |
| Phase Error Peak | 1, 62, 124 | 5, 10, 15 | -20 to +20 degrees |
| Frequency Error | 1, 62, 124 | 5, 10, 15 | -90 Hz to +90 Hz |
| Power Ramping | 1, 62, 124 | 5, 10, 15 | Mask |
| Modulation | 1, 62, 124 | 5, 10, 15 | Mask |
| Switching Transients | 1, 62, 124 | 5, 10, 15 | Mask |
| Power Reading |  |  |  |  |  |
| Output Power Average | 1, 62, 124 | Level 15 | 11.2 dBm to 14.8 dBm |
|  | 1, 62, 124 | Level 10 | 21.2 dBm to 24.8 dBm |
|  | 1, 62, 124 | Level 5 | 31 dBm to 34.1 dBm |

| RECEPTION |  |  |  |
| --- | --- | --- | --- |
| Rx Level | 1, 62, 124 | -83 dBm | +/-2.5 dBm |
| Rx Level | 1, 62, 124 | -60 dBm | +/-2.5 dBm |
| Rx Level | 1, 62, 124 | -40 dBm | +/-2.5 dBm |
| TCH LOOP |  |  |  |
| SENSITIVITY |  |  |  |
| BER | 1, 62, 124 | -85 dBm | 0% |
| FER | 1, 62, 124 | -85 dBm | 0% |
| BER | 1, 62, 124 | -102 dBm | < 2.44% |
| FER | 1, 62, 124 | -102 dBm | 0% |
| BER | 1, 62, 124 | -103 dBm | < 2.44% |
| FER | 1, 62, 124 | -103 dBm | 0% |

If a phone is out of the specifications, it must be sent to the Repair Center.

### PCN Specification (1800 Mhz)

| - Test Parameters |  | - Channel |  | - level | - Standard Specification |
| --- | --- | --- | --- | --- | --- |
| - EMMISSION |  |  |  |  |  |
| - Phase error RMS |  | - 512, 635, 760,885 |  | - 0,5,10 | - 0 to 5 degree |
| - Phase error Peak |  |  |  | - 0,5,10 | - -20 to +20 degree |
| - Frequency Error |  |  |  | - 0,5,10 | - -180 Hz to + 180 Hz |
| - Power Ramping |  |  |  | - 0,5,10 | - Mask |
| - Modulation |  |  |  | - 0,5,10 | - Mask |
| - Switching Transcients |  |  |  | - 0,5,10 | - Mask |
| - Power reading |  |  |  |  |  |
| - Output Power |  |  |  | - level 0 | - 30 +/- 2 dbm |
|  |  |  |  | - level 10 | - 10 +/- 4.0 dbm |
|  |  |  |  | - level 19 | - 0 +/- 5.0 dbm |

| - RECEPTION |  |  |  |
| --- | --- | --- | --- |
| - Rx Level | - 512, 635, 760,885 | - -100dbm | - 8 to 13 |
| - Rx Qual |  | - -100dbm | - 0 to 1 |
| - Rx Level | - 512, 635, 760,885 | - -80dbm | - 28 to 33 |
| - Rx Qual |  | - -80dbm | - 0 to 1 |
| - Rx Level |  | - -60dbm | - 48 to 53 |
| - Rx Qual |  | - -60dbm | - 0 to 1 |
| - TCH LOOP |  |  |  |
| - SENSITIVITY |  |  |  |
| - BER at -85dbm | - 512,635,760,885 |  | - 0% |
| - FER at -85dbm |  |  | - 0% |
| - BER at -103dbm |  |  | - 2.44% |
| - FER at -103dbm |  |  | - 0% |

If a phone is out of the specifications, it must be sent to the Repair Center.

## Charging IGN (Ignition) – Battery

### Plug the connector of the charger into the round socket set at the base of the transceiver.

The battery symbol should indicate the state of the charge process :

- Bars moving	- means the battery is being charged.

- Steady		- means the battery is fully charged.

If the battery is totally discharged, the battery icon will start scrolling 2 to 3 minutes only after being connected to charger.

### Remove the charger by unplugging the connector from the round socket at the base of the transceiver.

### Remove the battery.

### Lift the bottom left side of Production Test SIM Card with a tweezer.

### Gently slide the card out away from the grooves of the Product

# assembly / dismantlement procedures

During dismantlement and assembly operations, an antistatic bracelet must be used.

## Dismantlement

### Unscrew the ANTENNA

### Take the product, remove BATTERY

### Remove the SIM card

## Assembly

### Check the REAR HOUSING on the product

### Check the LABEL on the back

### Screw the ANTENNA

# Defaults Settings

## Reset customer parameters.

To reset customer parameters, it must use a GSM String. This Gsm String is

*#RSAV*#  or *#7728*#.

So, the defaults settings of the manufacturer are actived.

## Use of the GSM string *#RSAV*#  or *#7728*#.

Procedure to follow :

- Turn on the mobile (a SP SIM card is not necessary).

- Enter the Gsm String *#RSAV*#  or *#7728*#.

- You can see “Reset” but the customer parameters are not reset yet.

- Turn off the mobile.

- When you will turn on, the defaults settings will be actived.

# SOLUTIONS IN CASE OF PROBLEMS DURING THE TESTS

If for any reasons the phone needs to be disassembled (on level 2 only) to fix a defect detected during the test procedure, a complete functional test and a RF test must be done.

## The phone does not switch on.

- Check the tactile feeling of the “ON/OFF” button.

- Remove the battery. Check that both the contacts of the phone and those of the battery are not damaged.

- Clean the contacts.

- Plug the battery again, making sure that it is securely fitted. Charge the mobile until the icon has stopped flashing.

Then unplug from the charger and attempt to switch the mobile on.

If it still does not switch on, send the mobile for repair.

## Charge does not start or no detection of the charger.

- Check the charger contacts for dust or missing pins.

- Check the mobile connector.

- Remove the battery. Check that both the contacts of the phone and those of the battery are not damaged.

- Check the charger individually with a reference mobile. If the charger works properly try to charge the

customer mobile with a reference battery.

If  neither of the battery and the charger can be incriminated, send the mobile for repair.

## The display shows “No SIM card. Please insert your SIM card.” or  “SIM FAILURE”

- If the SIM card cannot be inserted, check for any foreign part and try to remove it.

- Check the SIM Card connector. All the contacts must be at the same level. Make sure that  there is no dust on the connector contacts and the SIM card contacts.

- If the test SIM card can be detected but the message “SIM Failure” remains on the customer’s card, his card must be damaged. Ask him to contact his network operator.

Otherwise send the mobile for repair

## Display problems

Contrast, icons and matrix of the display can be checked with the test SIM card by pressing  keys ”5”, “6” and “7” .

If everything  works in test configuration it then means that a phone setting is disabled or does not suit well. It can be solved in the phone menu.

Otherwise send the mobile for repair

## Buzzer problems

Buzzer tone can be checked with the test SIM card by pressing  key “1” and “2”.

- If it does not sound properly send the mobile for repair.

## No sound in Loudspeaker

The sound from the loudspeaker can be checked with the test SIM card by pressing key “3”.

- Check the microphone and the earpiece, If the failure cannot be found out, send the mobile for repair.

## Communication problems

- Sound quality can be checked in audio loop test (sound distortion, whistling, echo, …)

- If the mobile passes the radio tests successfully, we can assume that the phone works properly. The customer must check the coverage area of his network operator or that he does not use the phone in a radio shadow (outside the coverage area, in a tunnel or  between tall buildings, …)

- If the mobile does not pass the radio tests, send the mobile for repair.

## Defective antenna

- If the antenna is broken or curved => replace it

## Keyboard problems

-The keyboard can be checked with the test SIM card.

- If a key or a row does not respond, check the keyboard.

## Problems to send SMS messages

Check the Center number.  It may be empty or wrong.

# recommended PART LIST- tcd 989 XENIUM DB

## Common parts – out of warranty

| REFERENCE | DESIGNATION | POSITION | REPAIR LEVEL |
| --- | --- | --- | --- |
| 4311 257 61141 | Antenna ass’y X16 DB | 0906 | 1 |
| 4311 258 72846 | Battery Slim 800mAh (Li-on) | - | 1 |
| 4311 258 75075 | Vibra Battery (900 mAh Li-on) | - | 1 |
| 3122 427 20946 | Fast Charger Europe | - | 1 |
| 3122 427 21206 | Fast Charger UK | - | 1 |

ANNEXE 1

Each returned product must have an IRIS code to identify the failure.