Instructions manual for trace of the PTX427

Instructions manual for traces to use with PTX427 Heiji tool analyzer

Key words	:	Trace, Heiji, analyze, Layer3

Filename	:	/tmp/in/input.doc

Author	:	Andersen Ida. Sony PMC-Europe, Munich.

No. of Pages	:	5

1               xxxxx

Released	:	15. July 1998

Copyright

This is an unpublished work, the copyright of which vests in Sony International (Europe) GmbH, Personal and Mobile Communications Europe. All rights reserved. The information contained herein is confidential and the property of Sony International (Europe) GmbH, Personal and Mobile Communications Europe and is supplied “AS IS” without any warranty or liability. It may not be reproduced, disclosed or used in part or in whole except as authorized by contract with or written permission by Sony International (Europe) GmbH, Personal and Mobile Communications Europe. The foregoing provisions apply to all media in which the information may be embodied.

Document History

| Rev. | Changing Event | Released by | Date |
| --- | --- | --- | --- |

Table of Contents

1. Introduction

1.1 Executables

1.2 User instruction

i. References

ii. Abbreviations

iii. Distribution List

# Introduction

This document contains a description of how to trace and analyse signals within the PTX-427, using the analyser from the Z1 trace tool, Heiji. This also includes tracing signals between layers within the protocol stack.

## Executables

To be able to actually trace and analyse the Layer3 signals within the MS, a few executables are required. One is the trace.exe file, which is found in “..\gsm_ms\trace”. Another file is the analysing file, Heiji.exe, which can be found in “..\gsm_ms\c1heiji”.

To run the trace program, a shortcut must be made to the trace.exe file. The following command line options should be added:

- -p1 (com 1), -p2 (com2), etc. Compulsory

- -t (only on NT machines) Compulsory on NT machines, omit it on Win95 machines

- -f<tral230.ini> (another ini-file than tral230.ini can be used) Compulsory

The trace program generates a binary file c1trace.dat, which is then used as an argument to the analyser. The analyser, heiji.exe, has got the following commandline options:

- -i (analyse mode, with screen output) or -b (analyse mode, no screen output) Compulsory

- c1trace.dat (file output from the trace tool) Compulsory

## User instruction

Follow the step by step instructions below, to create the binary file for use in the analyser:

- First connect the MS to a PC using a serial connection, and setup the executables as described in Section 1.1.

- Execute the shortcut to the trace.exe and power on the mobile (any order is possible).

- Enter the CTE screen by typing ‘ (on an English keyboard), just above the right shift key.

- Enable the trace by entering <tren> (= trace enable). Note, that when enabling these traces, the Optimay traces will be disabled, to ensure performance of the MS, i.e. no overload. When the traces has been enabled, dots will run over the screen for every signal being traced. To be able to get the Optimay traces back, the trace program must be restarted (will be fixed later).

Now perform all the actions that needs to be traced. When the trace should be stopped, do as described below:

Either

- press escape twice to exit the trace tool,

or

- disable the traces by typing <trdis> (= trace disable)

Now the c1trace.dat file is in the “..\gsm_ms\trace” folder, and the analysing should be started.

- Copy/move the c1trace.dat file to the “..\gsm_ms\c1heiji” folder.

- Execute the shortcut to the heiji.exe file, with appropriate arguments (see Section 1.1)

- Finally a text file (trace.txt) is created in which all the signals are displayed, with references to the GSM recommendations.

IMPORTANT: Data is appended to the binary file, which means that the next trace will not be in a separate file, but at the end of the old trace. The text file generated by the c1Heiji tool, is on the other hand overwritten, so old files from other traces, must be renamed if they should be kept.

References

Trace tool for the PTX427.

Z1 trace tool Heiji.

Abbreviations

MS	Mobile Station

Distribution List